Introduction to 8086/8088 Microprocesser - General Facilities

- BIU and EU

- Data Registers

- Segment Registers

- Index Registers

- Pointer Registers

- Flag Register

- Memory Addressing

- Physical Memory Address Calculations.

8086 – Introduction

• 16 bit µp with 20 bit address bus & 16 bit data bus • Can address up to 220 = 1 MB memory directly • Can read or write 8 or 16 bit data

• 8088 - 8 bit data bus

• Internal architecture has two main units • BIU - Bus Interface Unit

• EU – Execution Unit

• BIU and EU work simultaneously for instruction  execution and form 2 stage instruction pipeline. • BIU – has bus interface logic, segment registers,  memory addressing logic and 6 byte instruction  queue.

• While EU is busy in instruction execution, BIU  fetches instruction from memory and stores them  in instruction queue.

• If EU executes a jump instruction

– Program control to be transferred to another location  then BIU

• Resets the queue

• Fetches instruction from new address

• Passes the instruction to EU

• Starts refilling the queue from new location • Process is called pipeline flushing.

BIU of 8088

• 8088 – 4 byte instruction queue

• Each time there is empty space of 2 bytes or more  8086 BIU startes filling the queue.

• 8088 BIU fills the queue when there is empty space  of 1 byte.

Execution Unit (EU)

• Contains – instruction decoder, ALU, general purpose  registers, pointer registers, index registers, flag register  and control circuitary to execute instruction. • EU is resposible for-

– Execution of instructions

– Providing address to BIU for fetching

data/instruction.

– Manipulating various registers as well as flag register. • EU is almost completly isolated from outside world. • BIU performs all bus operations for EU.

• Overlapped fetching and execution of instruction is  shown

It is assumed that

• Two µps i.e 2nd generation and 8086 execute the  instruction in same no. of clock cycles.

• Due to overlapping of fetch and excute, 8086/8088  is able to excute 3 instruction in lesser time but also  able to fetch 4th and 5th instruction.

• Register Set- 8086 has fourteen – 16 bit registers  • Data Registers

• Segment Registers

• Pointer & index register

• flag register

• Data Registers- Four sixteen bit AX,BX,CX,DX • Or 8 – 8 bit registers

• AH AL BH BL CH CL DH DL

• AX- Primary Accumulator

• Input-Output operations through AX(or AL) • Several string operations require one operand  to be in AX (or AL).

• 16 bit divide or multiply - AX has one word  operand (AL for 8 bit operation).

• 32 bit multiply or divide – AX holds lower order  word operand.

• BX – GPR (Genral Purpose Register) + Base Register • CX – GPR + Count Register in multi iteration  instruction.

• DX – GPR + used in I/O instructions, multiply  & divide instructions.

Segment Registers

• concept of memory segmentation.

Memory divided into no. of parts called segments. • In 8086, 1 MB physical memory can be divide into 4  types of segments.

• Starting address of each segment is placed in 16 bit  register

• CS – Code Segment Register.

• DS – Data Segment Register.

• SS – Stack Segment Register.

• ES – Extra Segment Register.

• Each segment has memory space of 64k byte

Index Registers

SI – Source Index Register

DI – Destination Index Register

• May be used as GPR. However main purpose is to  store address offset in case of – Indexed, Base  Indexed, and Relative Base Indexed addressing  modes

Pointer Registers

• Stack Pointer(SP) - provides access to stack top.  Holds offset address of stack top.

• Instruction Pointer(IP) - holds offset address of next  instruction to be executed. Like program counter. • Base Pointer (BP) - GPR. Main purpose is to provide  indirect access to data in stack.

• Flag Register - also called program status word  (PSW)

9 active Flags

6 status flags

3 control flags

Status Flag –

• Carry – Arithmetic operation results is carry out of  MSB or Borrow in Subtraction. Also used in shift &  Rotate.

• Parity – Even parity in byte operation or lower byte  of word operation

• Auxiliary Carry – set if there in carry out of lower  nibble to higher nibble in 8 bit quantity or lower  byte to higher byte in 16 bit quantity. Used in BCD  operations.

• Zero – set when result of operation is zero.

• Sign – set when after arithmetic or logic  operation, MSB of result = 1 indicates that result is  negative.

• Overflow – used to detect magnitude overflow in  signed arithmetic.

• For addition operation – flag is set when there  is carry in to MSB but no carry out of MSB or  vice versa.

• For subtraction – flag is set when MSB needs a  borrow but there is no borrow from MSB or  vice versa.

Control Flags –

• Direction – Used with string operations. When set  (i.e = 1) then string operation in auto decrement  mode (i.e right to left) when reset (i.e = 0) then  string operation in auto increment mode (left to  right)

Interrupt Enable –

• When = 1, enables 8086 to recognize external  interrupts. When = 0, all maskable interrupts are  disabled. No effect on non maskable interrupts.

• Trap flag – when = 1, sets processor in single step  mode for debugging.

MemoryAddressing– RealMode

• Real Mode – limited to 1 MB of memory i.e real  memory.

• Segment Register – 16 bits in length.

• Memory address – 20 bits.

• Segment register content are appended with 0000 to  make it 20 bit address i.e segment register contents are  multiplied by 16

• Now the address represent the starting address of  segment in memory

• Let us say CS = 1234H

• After appending with 0000(=0H) we get 12340H as  starting address of code segment in memory. • The size of a segment i.e 64kb i.e FFFFH

• The address range of code segment 12340H to (12340  + FFFF) = 2233FH

• A segment must thus start at 16 byte boundary  [since last 4 bits = 0000]

• Two segment can overlap with each other partially  if segment address have been defined like that. • There may be more then one segment of a  particular type. For that segment register contents  are dynamically changed.

• All real mode memory addresses consist of segment  address plus an offset address.

• Offset address – increment from starting of  segment.

• Thus segment address defines the starting address  of 64k byte memory segment.

• Offset address selects a location within 64k byte  memory segment.

• Offset address is also known as logical address. • Physical address is actual memory location of  operands = segment address appended with 0H +  offset address = (segment address) x 16 + offset  address.

- Let us take an example.

-> Segment register = 1234H

-> Offset = 0022H

Physical address ?

Segment starting address = 12340H

+ offset = 0022H

________________________

Physical address = 12362H

Exercise 1 – calculate last address of segment, given  the segment register values as 1234H, 2300H and  AB00H.

Exercise -2 – Given physical address of memory  location and segment register content, find the  logical address i.e offset

Note –

• 8086 and 8088 operate only in real mode • 80286 – 80486 – Pentium – operate in real or  protected mode.

• Real mode operation allows microprocessor to  address only first 1 MB of memory space even if it is  80486 µp.

• Memory segmentation allows program to be  relocated in any portion of memory.

• There is a set combination of segment register and  offset register known as default combination . Segment offset Special purpose CS IP Instruction address SS SP or BP stack address DS BX,DI,SI Data address An 8 or 16 bit number

ES DI for string string destination instruction address