PROJECT: OPCODE FINDER

Steps:

A. FIND OPCODE OF INSTRUCTION:

1. Check MOV, ADD

2. Check first operand

a. does it have brackets or no brackets until ','

a1. what is inside brackets

b. what is the operand

c. is it 16bit operand or 8bit operand

3. Check second operand

a. does it have brackets or no brackets

a1. what is inside brackets

b. What is the operand

c. is it 16bits operand or 8bits operand

d. Check the displacement

4. Find reg code

5. Find r/m code

6. Find mod code

7. Find D

8. Find w

9. Find disp

a. check if it is 16bits displacement or 8bits displacement

b. break it into two bytes if it is 16bits

10. Write all bits in the 32bits

11. Breakdown these into four parts of 8bits

12. Form the opcode

B. PERFORM THE INSTRUCTION USING MEMORY

C. OUTPUT WHAT IS EXACTLY HAPPENED IN THE CPU AND INSTRUCTION EXECUTION

Information

General Purpose Registers:	Segment Registers	Special Purpose Registers

AX				CS			SP

BX				DS			BP

CX				ES

DX				SS

IP

Addressing Modes

1. Register Addressing

MOV AX, BX

2. Immediate Addressing

MOV AX, 1420H

3. Direct Addressing

MOV AX, [2340H]

4. Register Indirect Addressing

MOV AX, [BX]

5. Base-plux-Index Addressing

MOV AX, [BX+SI]

6. Register Relative Addressing

MOV AX, [BX+40]

7. Base relative-plus-indexed Addressing

MOV AX, [BX+SI+10]

8. Scaled Indexed Addressing: Not possible in 8086

8086 Assembly Language to Machine Language

Byte1   1 0 0 0 1 0 D W

Byte2   MOD REG R/M

Byte3   LowDisp

Byte4   HighDisp

Byte5   Dir Addr LB

Byte6   Dir Addr HB

D =>    1 Data flow to the REG field from R/M

0 Data flow from the REG field to the R/M

W =>    1 16bits

0 8bits

MOV

Register/Memory to/from Register    1 0 0 0 1 0 d W mod reg r/m

Immediate to Register/Memory        1 1 0 0 0 1 1 W mod 0 0 0 r/m data data if w 1

Immediate to Register               1 0 1 1 w reg data data if w 1

Memory to accumulator               1 0 1 0 0 0 0 w add-low add-high

Acumulator to Memory                1 0 1 0 0 0 1 w add-low add-high

Register/Memory to SegmentRegister  1 0 0 0 1 1 1 0 mod 0 reg r/M

SegmentRegister to Register/Memory  1 0 0 0 1 1 0 0 mod 0 reg r/M

AX 000			AL 000			ES 00

CX 001			CL 001			CS 01

DX 010			DL 010			SS 10

BX 011			BL 011			DS 11

SP 100			AH 100

BP 101			CH 101

SI 110			DH 110

DI 111			BH 111

mod 00: disp = 0 except for 16bit displacement when r/m = 110

mod 01: 8bits disp & disp high is absent

mod 10: 16 bits disp

mod 11: Register Mode (no displacement) r/m is treated as reg field

if r/m == 000 then EA == (BX) + (SI) + DISP

if r/m == 001 then EA == (BX) + (DI) + DISP

if r/m == 010 then EA == (BP) + SI + DISP

if r/m == 011 then EA == (BP) + (DI) + DISP

if r/m == 100 then EA == (SI) + DISP

if r/m == 101 then EA = (DI) + DISP

if r/m == 110 then EA == (BP) + DISP

if r/m == 111 then EA == (BX) + DISP

MOV [AX], [BX + SI + DISP]

1 0 0 0      1 0 1 1        1 0 0 0         0 0 0 0

8            B              8               0